Campus Network Configuration

🌐 Project Overview

This project involves the complete configuration of a large-scale hierarchical network using Cisco Packet Tracer. The topology simulates a university-like campus network with multiple blocks and floors, connected via routers, switches, firewalls, and a central core.

🔧 Components Used:

- Main Core Router and Main Firewall

- Campus Routers: KIET, KIEK, KIETW, E-Block

- Block Routers (per floor)

- Cisco ASA Firewalls

- Switches (per floor)

- PCs (DHCP clients)

📊 Logical Topology:

[Main Core Router] <--> [Main Firewall]

|

----------------------------------------------------

|                |                 |              |

[KIET]           [KIEK]          [KIETW]         [E-Block]

|                 |                 |              |

[F0 Routers]  [F0 Routers]     [F0 Routers]     [F0 Routers]

|                 |                 |              |

[Switches]     [Switches]       [Switches]       [Switches]

|                 |                 |              |

[F1–F4 Routers] ... connected via switches ...

⚖️ IP Addressing Scheme

- Router-to-Router / Firewall Links: /30

- Internal Subnets (PCs): /24

- IPv6: Used with FD00::/8 range per floor

🚧 VLAN Configuration

Each switch was configured with VLAN 10 for student PCs:

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name STUDENT

Switch(config-vlan)# exit

Switch(config)# interface range fa0/3 - 0/7

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

🔢 DHCP Configuration (on Floor Routers)

DHCP was configured on each floor router to provide IP addresses to the PCs connected via access switches. The routers were assigned internal interfaces (e.g., Fa0/1) and DHCP pools were defined for each subnet.

📁 DHCP Configuration Example:

Router> enable

Router# configure terminal

Router(config)# ip dhcp excluded-address 10.0.1.1 10.0.1.10

Router(config)# ip dhcp pool FLOOR1_POOL

Router(dhcp-config)# network 10.0.1.0 255.255.255.0

Router(dhcp-config)# default-router 10.0.1.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

📂 Interface Configuration (For DHCP to Work):

Router(config)# interface fa0/1

Router(config-if)# ip address 10.0.1.1 255.255.255.0

Router(config-if)# no shutdown

✅ On Each PC:

- Go to Desktop > IP Configuration

- Select DHCP

- It should automatically get an IP in the range, e.g., 10.0.1.11, with default gateway 10.0.1.1

✊ Troubleshooting:

- Check if interface is UP (show ip int brief)

- Ensure the DHCP pool matches the interface subnet

- Check switch VLAN if PCs are in correct VLAN 10

🌎 IPv6 Configuration

Router(config)# interface fa0/0

Router(config-if)# ipv6 address FD00:BLOCK:A1::1/64

Router(config-if)# ipv6 enable

Router(config-if)# no shutdown

PCs set to auto-configure IPv6 and verify via ping.

⚖️ OSPF Configuration (Floor Routers & Campus Router)

Router(config)# router ospf 1

Router(config-router)# router-id 1.1.1.1

Router(config-router)# network 192.168.10.0 0.0.0.3 area 0

Router(config-router)# exit

Campus router has OSPF entries for all its connected floor router subnets.

🛡️ BGP (Demo Configuration on Main Core)

Router(config)# router bgp 65000

Router(config-router)# neighbor 192.168.200.2 remote-as 65001

Router(config-router)# network 192.168.100.0 mask 255.255.255.252

Used for simulation between main core and campuses.

🔐 ACL Configuration

Router(config)# access-list 100 permit tcp any any eq 80

Router(config)# interface fa0/0

Router(config-if)# ip access-group 100 in

🚗 NAT Configuration (Campus Routers)

Router(config)# access-list 1 permit 10.0.0.0 0.0.255.255

Router(config)# ip nat inside source list 1 interface fa0/0 overload

Router(config)# interface fa0/1

Router(config-if)# ip nat inside

Router(config)# interface fa0/0

Router(config-if)# ip nat outside

⏱️ NTP Configuration

Router(config)# ntp server 192.168.1.100

All routers sync to the main time server.

🚼 QoS Configuration (Basic Policy)

Router(config)# class-map match-any VOICE

Router(config-cmap)# match ip dscp ef

Router(config)# policy-map QOS-POLICY

Router(config-pmap)# class VOICE

Router(config-pmap-c)# priority percent 70

Router(config)# interface fa0/0

Router(config-if)# service-policy output QOS-POLICY

🌐 Spanning Tree Protocol (STP)

Switch(config)# spanning-tree mode pvst

Switch(config)# spanning-tree vlan 10 priority 4096

Primary STP root is defined on one main switch per block.

👥 PC Configuration

Each PC:

- Set to DHCP for IPv4

- Auto-configure for IPv6

- Test connectivity using ping and tracert

✅ Final Testing Steps

- Ping between PCs across floors and blocks

- Verify OSPF neighbors: show ip ospf neighbor

- Check routing tables: show ip route

- IPv6 status: show ipv6 interface brief

- NAT translations: show ip nat translations

- Check ACL hits: show access-lists

🎉 Conclusion

This project demonstrates the successful configuration of a complex hierarchical network with full Layer 3 routing, IPv4/IPv6 support, security policies, VLAN segmentation, and more using Cisco technologies.

Well done! 🚀