Networking

Practical learning   :

- Assigning IP addresses to the interfaces.

- Making subnets in any type of IP classes

- Configuration of routing protocols

RIP

OSPF

EIGRP

BGP

- Configuration of VLANS and its types

* Assigning IP addresses to the interfaces   :

➤ What it means:

Assigning an IP address allows a device to communicate in a network. Each interface (like Ethernet0, FastEthernet1, etc.) in a router or switch needs a unique IP address.

➤ Steps:

192.168.1.2 → IP address of the PC

255.255.255.248→ Subnet mask

192.168.1.1 → Default gateway

=> Verify the configuration:   show

=> Test connectivity:  ping 192.168.1.1

Configuration of Routers:

enable

configure terminal

interface fastEthernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

Similarly for R2 and R3

RIP : (Routing Information Protocol ) :-

A distance-vector protocol that uses hop count as a metric.
Slow convergence and supports up to 15 hops.
Simple but outdated, mostly used for learning purposes.

Steps:

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.168.1.0

Router(config-router)# exit

Similarly for the router 3

➤OSPF (Open Shortest Path First) :-

A link-state protocol that uses cost based on bandwidth.
It converges quickly and supports large, hierarchical networks.
Open standard, widely used in enterprise networks.

➤Steps :

Router(config)# router eigrp 10

Router(config-router)# network 192.168.1.0

Router(config-router)# no auto-summary

Router(config-router)# exit

➤ EIGRP (Enhanced Interior Gateway Routing Protocol) :

A Cisco proprietary hybrid protocol (distance vector + link state features).
Fast convergence and efficient use of bandwidth.
Supports VLSM, unequal-cost load balancing, and is scalable.

➤Steps :

Router(config)# router eigrp 10

Router(config-router)# network 192.168.1.0

Router(config-router)# no auto-summary

Router(config-router)# exit

➤ BGP (Border Gateway Protocol) :

A path-vector protocol used to route between autonomous systems (on the internet).
Used by ISPs and large networks for global routing decisions.
Scalable and policy-based, but complex to configure.

➤Steps:

Router(config)# router bgp 65001

Router(config-router)# neighbor 10.0.0.2 remote-as 65002

Router(config-router)# network 192.168.1.0 mask 255.255.255.0

Router(config-router)# exit

4. Configuration of VLANs and Its Types

➤ What is a VLAN?

- Virtual LAN allows segmentation of a physical network into multiple logical networks.

- Devices in different VLANs can't communicate without a router or Layer 3 switch.

➤ Types of VLANs:

- Default VLAN (VLAN 1) – All switch ports are in this by default.

- Data VLAN – Carries user-generated traffic.

- Voice VLAN – Used for VoIP traffic.

- Management VLAN – Used to manage network devices.

- Native VLAN – Used for untagged traffic on trunk ports.

➤ Steps to Create and Assign VLAN:

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name Sales

Switch(config-vlan)# exit

Switch(config)# interface fastEthernet 0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

➤ To Configure Trunk Port (for multiple VLANs):

=Switch(config)# interface fastEthernet 0/24

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk native vlan 1