Week-2

1. WLAN (Wireless Local Area Network)

Definition:
WLAN is a wireless network that connects devices using radio frequency instead of wired connections. It enables mobile and fixed devices (like laptops, phones, and printers) to access the internet or local network resources within a limited area such as homes, schools, or enterprises.

Key Components & Concepts:

- SSID (Service Set Identifier):
A unique identifier or name of the wireless network that devices use to connect.

- BSSID (Basic Service Set Identifier):
The MAC address of the Access Point (AP) broadcasting the SSID.

- Channel:
A specific frequency range used to transmit signals. Interference is avoided by selecting non-overlapping channels (e.g., 1, 6, 11 in 2.4GHz).

- Bandwidth:
The capacity or rate at which data can be transferred, typically measured in Mbps.

- Roaming:
The process by which a device seamlessly switches from one AP to another with stronger signal strength within the same SSID.

Security Protocols:

| Protocol | Encryption | Status |
| --- | --- | --- |
| WEP (Wired Equivalent Privacy) | Weak (static key) | Deprecated |
| WPA (Wi-Fi Protected Access) | Better than WEP | Rarely used |
| WPA2 | Strong (AES encryption) | Standard |
| WPA3 | Strongest with forward secrecy | Emerging |

Example:
In our college, the “KIET_WIFI” is a WLAN. Students in various blocks or floors connect to different APs with the same SSID. This allows roaming and stable connectivity.

2. IPv6 (Internet Protocol Version 6)

Definition:
IPv6 is the latest version of the Internet Protocol that solves the limitations of IPv4, especially the exhaustion of address space. It enables the growth of the Internet and supports more advanced networking features.

Key Features:

- 128-bit Addressing:
Provides ~3.4×10³⁸ possible addresses, allowing unique IPs for every device on Earth (and beyond).

- No Need for NAT:
Every device gets a unique public IP, avoiding address translation.

- Built-in Security (IPsec):
Mandatory support for secure communications.

- Auto-configuration (SLAAC):
Devices can configure themselves automatically using Router Advertisements.

- Simplified Header Format:
Reduces processing overhead on routers.

IPv6 Address Types:

| Type | Usage |
| --- | --- |
| Unicast | One-to-one |
| Multicast | One-to-many |
| Anycast | One-to-nearest |

Address Example:

- Full: 2001:0db8:0000:0000:0000:ff00:0042:8329

- Shortened: 2001:db8::ff00:42:8329

Real-world Use:
With billions of IoT devices such as smart bulbs, fridges, or cameras, IPv6 becomes crucial to assign unique addresses to each without complex NAT setups.

3. STP (Spanning Tree Protocol)

Definition:
STP is a Layer 2 protocol that prevents broadcast storms and loops in a switched Ethernet network with redundant paths by selectively blocking links.

How STP Works:

- Root Bridge Election:
All switches elect a root switch based on the lowest Bridge ID.

- Root Port Selection:
Each non-root switch selects one port with the shortest path to the root.

- Designated Port:
The best port on each segment is designated for forwarding traffic.

- Blocking Ports:
Any redundant port is put into a blocking state to prevent loops.

STP States:

- Blocking: No data forwarding; listens for BPDUs.

- Listening: Checks for loops but no learning or forwarding.

- Learning: Learns MAC addresses.

- Forwarding: Forwards data frames.

- Disabled: Admin or error shutdown.

Faster Versions:

- RSTP (Rapid STP – 802.1w): Faster convergence (<1 second).

- MSTP (Multiple STP – 802.1s): Allows multiple VLANs to use separate spanning trees.

Example:
Three switches form a triangle. STP will block one of the links to prevent a loop. If a working link fails, STP re-enables the blocked one.

4. CDP and LLDP

CDP (Cisco Discovery Protocol):

- Cisco proprietary protocol used to discover directly connected Cisco devices.

- Works at Data Link Layer (Layer 2).

- Shows information like device ID, interface, platform, IP address.

LLDP (Link Layer Discovery Protocol):

- IEEE 802.1AB standard (vendor-neutral).

- Helps in identifying devices in the same Layer 2 network.

- Often used for VoIP, switches, and network inventory.

Use Cases:

- Network topology discovery.

- Detecting miswired cables.

- Device identification in large networks.

Example:
Network admins use CDP/LLDP to see which devices are connected to a particular switch port, which helps in troubleshooting.

5. ACL (Access Control List)

Definition:
ACLs are a set of rules that control incoming or outgoing network traffic. They operate at Layer 3/4 and are used to permit or deny traffic based on IP, protocol, or port.

Types of ACLs:

| Type | Filters On | Example Command |
| --- | --- | --- |
| Standard | Source IP only | access-list 10 permit 192.168.1.0 0.0.0.255 |
| Extended | Source + destination IP + port | access-list 100 deny tcp any any eq 23 |

Placement:

- Inbound ACLs: Check packets before entering the interface.

- Outbound ACLs: Check packets before leaving the interface.

Use Case Example:
To allow only Admin PCs (192.168.1.0/24) to access a file server (10.10.10.10), an ACL is applied on the router interface leading to the server.

6. NTP (Network Time Protocol)

Definition:
NTP is used to synchronize time between computers in a network, ensuring all devices have accurate and consistent time.

Why is it Important?

- Log timestamps are accurate and aligned.

- Certificate validity depends on correct time.

- Time-sensitive operations like backups, authentication, or security alerts rely on synchronized time.

Stratum Levels:

| Stratum | Description |
| --- | --- |
| 0 | Reference clocks (e.g., atomic, GPS) |
| 1 | Directly connected to Stratum 0 |
| 2 | Syncs from Stratum 1 |

Example:
Firewall logs and system logs can be correlated only if all devices sync time via NTP, especially during incident response.

7. NAT (Network Address Translation)

Definition:
NAT is a technique used by routers to map private IP addresses to public ones, enabling internal devices to access external networks (like the internet).

Types of NAT:

| Type | Description |
| --- | --- |
| Static NAT | One-to-one mapping between internal & external IP |
| Dynamic NAT | Internal IPs mapped from a pool of public IPs |
| PAT (NAT Overload) | Many-to-one mapping using different ports |

Why NAT?

- Conserves IPv4 address space.

- Provides basic security by hiding internal network structure.

Example:
In a home network, multiple devices use private IPs like 192.168.0.x, but the ISP assigns only one public IP (e.g., 49.205.112.54). NAT handles this translation.

8. Firewalls

Definition:
A firewall is a network security device (hardware or software) that monitors and controls incoming and outgoing traffic based on pre-defined rules.

Types of Firewalls:

| Type | Functionality |
| --- | --- |
| Packet-filtering | Basic IP, port, and protocol-based filtering |
| Stateful Inspection | Tracks connection states (e.g., TCP handshake) |
| NGFW (Next-Gen) | Application layer filtering, deep inspection, and threat detection |
| Host-based | Software on end devices (e.g., Windows Firewall) |

Common Rules:

- Allow: HTTP (80), HTTPS (443), DNS (53)

- Deny: Telnet (23), FTP (21) from outside

- Block: Malicious IPs or unknown ports

Example:
Colleges block gaming or torrent websites using firewalls to prevent bandwidth hogging and ensure academic productivity.

9. QoS (Quality of Service)

Definition:
QoS is a set of techniques to manage network resources by prioritizing traffic based on type, source, or destination. It's crucial for latency-sensitive applications like video calls, VoIP, and online gaming.

QoS Mechanisms:

| Mechanism | Role |
| --- | --- |
| Classification | Identify traffic types (e.g., video, voice) |
| Marking | Assign DSCP or CoS values to packets |
| Queuing | Arrange packets in priority queues |
| Shaping | Delay lower-priority traffic to reduce congestion |
| Policing | Drops traffic that exceeds a configured threshold |

Example:
During online classes, Zoom calls should get higher priority over students downloading large files, ensuring smooth communication.

Final Summary Table

| Topic | Key Concept | Practical Use | Real-world Example |
| --- | --- | --- | --- |
| WLAN | Wireless network access | Connect mobile devices | Student connects phone to campus Wi-Fi |
| IPv6 | 128-bit addressing | Future-proofing internet | Smart devices at home using unique IPs |
| STP | Loop prevention | Stable Ethernet topology | Switch blocks loop in ring network |
| CDP/LLDP | Device discovery | Network mapping | Admin sees devices and connections clearly |
| ACL | Traffic filtering rules | Security enforcement | Block FTP from guest network |
| NTP | Time synchronization | Accurate logging | Matching logs from different devices |
| NAT | IP address translation | Share public IP | Home router uses NAT for internet access |
| Firewall | Traffic control | Protect internal network | Blocking harmful sites in college network |
| QoS | Bandwidth prioritization | Improve service quality | VoIP works without lag even during high usage |